NANA BAMBI'S PLACE

Hub Site Build — Complete Design & Costing Explanation

Ragay, Camarines Sur, Bicol  |  March 2026  |  Version 3

# 1. What We Are Building

A double-height U-shaped building on the hub site at Ragay. Ground floor: butcher shop, cold storage, operations. Upper floor: Alejandro's Cafe — a full restaurant with show kitchen, wraparound balcony, and food forest courtyard below. Plus a 3-bedroom manager's residence attached at upper floor level behind the earth rampart.

Designed from Aido's 5 years at Treetops Restaurant (Travelodge Mirambeena, Darwin) and 2 years at Frontier Hotel Darwin Rooftop Restaurant.

# 2. Design Concept — Hybrid Structure

The building uses the right material for each job. Ground floor rear: reefer containers buried in earth rampart for cold storage. Ground floor front: CHB block walls for butcher and ops. Upper floor: light steel frame on the container ring with nipa roof over sealed membrane. The result is unmistakably Filipino — a massive nipa structure with containers doing the hard work invisibly.

| Element | Material | Why |
| --- | --- | --- |
| Ground floor rear | 4x reefer hi-cube containers | Cold rooms + freezers — passive insulation in rampart |
| Ground floor front | CHB 150mm rendered | Butcher + ops — local trades, local materials |
| Upper floor spine | 2x reefer hi-cube (20ft + 40ft) | Kitchen back wall + structural ring |
| Upper floor frame | Light steel + FCB deck | Open, airy, tropical restaurant |
| Roof | Torch-on membrane + nipa overlay | Sealed waterproof with traditional appearance |
| All glazing | UPVC frames + toughened glass | Zero maintenance, typhoon rated, no rust |
| All floors | Tiled — porcelain throughout | Durable, easy clean, slip-rated |
| Residence | Light steel + FCB + nipa on poles | Matches hub aesthetic |

# 3. Container Frame — U-Shape

Six reefer hi-cube containers form the horseshoe. All purchased as insulated shells — compressor units removed. Remote refrigeration installed separately.

| Element | Specification |
| --- | --- |
| Rear bar | 1x 20ft + 1x 40ft = 18.25m wide |
| Wings | 2x 40ft, one each side = 12.192m deep |
| Breezeway | 3.0m gap |
| Total depth | 15.192m |
| Clear span | 13.374m |
| Container height | Hi-cube 2.690m internal |

Why all reefers: Better insulation throughout — not just cold rooms. Upper floor kitchen spine is naturally cooler. Used reefers with dead compressors are the cheapest category at container yards.

# 4. Earth Rampart — 5 Functions

The single most important engineering element. One structure — five jobs.

- 1. PASSIVE INSULATION — earth mass keeps buried reefers cold, cuts refrigeration costs

- 2. ACCESS RAMP — 1:10 grade, 29m x 4m. Guests walk up through garden. No stairs. Forklift and trolley rated.

- 3. WATER STORAGE — 3x 30,000L = 90,000L tanks on top, gravity-fed to building. No pumps for normal supply.

- 4. HIDDEN INFRASTRUCTURE — all tanks concealed. Clean aesthetic.

- 5. PRODUCTIVE GARDEN — walled trellis garden over tanks. Kitchen herbs and fruit. Chef picks from rear door.

# 5. Roof — Torch-On Membrane + Nipa

Looks native. Fully sealed. Five layers:

- Layer 1: Steel rafters + purlins (structure)

- Layer 2: 19mm FCB screwed to purlins (rigid substrate)

- Layer 3: Torch-on bitumen membrane x 2 layers (THE waterproofing — fully sealed)

- Layer 4: 50mm batten grid (air gap — nipa breathes and dries, prevents rot)

- Layer 5: Nipa panels (finish — replaceable section by section without touching membrane)

From inside guests see nipa. From outside it's a traditional nipa building. The membrane means it never leaks.

# 6. Glazing & Doors — All UPVC + Toughened Glass

Every window, every door, every glass panel: UPVC frames with toughened glass. Zero maintenance, no painting, no rust, no termites. This is typhoon country — laminated toughened glass on the bifold doors.

## Bifold Doors — The Defining Feature

Full-length UPVC bifold doors on both wing inner faces (12.192m each). When fully open, interior dining merges seamlessly with the wraparound balcony. This is the Darwin rooftop experience.

## Full Glass Wrap

- UPVC French doors — front face centre entry with fixed glass sidelights

- UPVC fixed windows — front face either side of French doors

- UPVC glass pass-through — full width above show kitchen sub-wall

- UPVC awning windows — above bifolds for ventilation, fly-screened

- UPVC fixed panels — gable ends to match roof pitch

## Ground Floor

- UPVC awning windows — butcher area (fly-screened, toughened)

- UPVC awning windows — ops area (fly-screened, toughened)

- UPVC entry door — butcher customer entrance (self-closing)

- UPVC service door — ops receiving

## Balcony Railings

Hardwood posts + stainless steel cable railings on all 3 sides. Unobstructed view down to food forest courtyard. Bolted to cantilever frame.

# 7. Cold Chain Spine

Reefers (buried rear) -> Butcher (ground floor) -> Dumbwaiter -> Rear Kitchen (upper floor)

Vertical and tight — no long warm runs. Four rooms: 2x cool room (2-4 C), 2x freezer (-18 C). All remote standalone refrigeration — easier to service, more efficient, replaceable. Equipment in shaded service yard for heat rejection.

# 8. Alejandro's Cafe — Upper Floor

| Zone | Area | Use |
| --- | --- | --- |
| Rear kitchen (production) | 73.6m2 | Combi oven, grill, hob, brat pan, fryers, prep benches |
| Show kitchen | 28m2 | Charcoal char grill centrepiece, display cabinets — guest chooses cut |
| Interior dining | ~101.5m2 | 12x 4-top tables, 48 seats, wood-look porcelain tile floor |
| Wraparound balcony | ~119m2 | 6x 4-top tables, 24 seats, anti-slip tile, string lights |
| Bar | 4.5m L-shape | 8 stools, 2 undercounter fridges, ice machine, cashless POS |
| TOTAL DINING | ~220.5m2 | 80 covers total |

## Kitchen Equipment Strategy

Secondhand ex-restaurant equipment from Cartimar Market (Pasay), Divisoria surplus dealers, ex-hotel auctions Manila. Allow 4-6 weeks lead time for transport to Ragay. Kitchen hood fire suppression system mandatory.

## Aquarium + Herb Garden Centrepiece

Custom freshwater aquarium with integrated live herb garden. Divides restaurant almost in two — each side can be split for separate events. Herbs connect to rear trellis garden supply. Living display, not a service station.

# 9. Butcher Fit-Out — Ground Floor

Retail butcher + prep room. NOT a slaughterhouse — carcasses arrive whole or quartered. Slaughterhouse is a 5-year plan, separate build, separate site.

- All tiled — floor (anti-slip porcelain R12) and walls to 2.1m (white ceramic). Epoxy grout throughout.

- Stainless prep benches, butcher's block, band saw, meat grinder, slicer, vacuum sealer

- Refrigerated glass display cabinet — customer-facing

- Foot-operated handwash (mandatory), boot wash tray at entry

- PVC strip curtains on cool room entries — retain cold, allow trolley pass-through

- Dumbwaiter sends prepped cuts up to Alejandro's kitchen

- All equipment secondhand from Manila surplus where possible

# 10. Manager's Residence

117.3m2 (17m x 6.9m) elevated on steel poles behind the earth rampart. Light steel frame, FCB, nipa cladding — matches hub aesthetic. 3 bedrooms, 2 bathrooms, laundry. No kitchen — residence connects directly to hub building at same level.

Plan based on Selecta Homes design (Port Lincoln SA, 2023) that Aido designed and built. Every spec is known.

Tax: Company-provided housing, CEO salary package. FBT at 35% grossed-up (modest at provincial rates). Corporate asset depreciating 20 years. Accountant to structure.

# 11. Build Sequence

| Phase | Work |
| --- | --- |
| 1 | Site prep — clear, grade, drainage channels, raised pad |
| 2 | Container delivery — all 6 reefer hi-cubes positioned by crane |
| 3 | Earth rampart — fill, compact, install water tanks on top |
| 4 | Structural steel — weld containers, floor joists, balcony outriggers |
| 5 | Roof — steel structure + torch-on membrane + batten grid + nipa |
| 6 | CHB ground floor walls — local trades |
| 7 | UPVC glazing + bifold doors installed |
| 8 | Electrical + plumbing rough-in |
| 9 | Refrigeration — remote units, pipework, commissioning |
| 10 | Tiling — restaurant floor, balcony, butcher walls + floor, kitchen |
| 11 | Kitchen fit-out + butcher fit-out |
| 12 | Restaurant fit-out — bar, furniture, aquarium, lighting |
| 13 | Residence — on poles, can run parallel from Phase 6 |
| 14 | Fire safety + security |
| 15 | Commissioning — test all systems, temperature logs, safety checks |

# 12. Build Cost Summary

| Section | Cost PHP |
| --- | --- |
| Containers — 6x reefer hi-cube | 1,230,000 |
| Ground floor slab (277m2) | 465,954 |
| Driveway + car park (limestone) | 503,600 |
| Drainage system | 107,000 |
| Building structure — CHB + light steel | 1,684,308 |
| Roof — torch-on + nipa | 1,693,528 |
| Earth rampart + access ramp | 466,400 |
| External stairs + dumbwaiter | 250,000 |
| Water system | 418,000 |
| Electrical & power | 627,000 |
| Cold chain — refrigeration | 685,000 |
| Fire safety + security + drainage | 438,000 |
| Glazing + UPVC doors + railings | 1,544,000 |
| Kitchen fit-out | 947,760 |
| Restaurant fit-out | 1,364,050 |
| Butcher fit-out | 554,300 |
| Manager's residence | 1,390,597 |
| TOTAL BUILD COST | 13,369,497 |
| Excl. land (4.5M in Trust) | 8,869,497 |

All values March 2026 estimates in Philippine Peso. Get 3 quotes minimum for every major line item. See NanaBambis_Build_Costing_v3.xlsx for full editable workings.

# 13. Outstanding Decisions

| Item | Decision Needed |
| --- | --- |
| Structural engineer | Sign-off: balcony cantilever, container welds, upper floor load rating |
| Dumbwaiter position | Which end of rear kitchen — left or right? Confirm before slab pour |
| UPVC colour | White or dark grey frames? Dark grey disappears better but costs ~10% more |
| Container height confirmation | Hi-cube confirmed at 2.690m — verify with supplier before purchase |
| Slaughterhouse | 5-year plan — separate build, separate site, separate costing |

"Use what you have. Build what lasts. Leave something worth inheriting."

Nana Bambi's  ·  Ragay, Camarines Sur, Bicol