NANA BAMBI'S

Equipment Acquisition Brief

Kubota MX5400 -- Primary Farm Tractor

Ragay, Camarines Sur, Philippines  |  2026

# 1. Purpose

This brief covers the tractor specification, attachment stack, sourcing strategy, pricing, and inspection checklist for the Nana Bambi's primary farm tractor. The selected machine is the Kubota MX5400 4WD HST with LA1065 front end loader and 4-in-1 bucket, sourced secondhand from Japan.

# 2. Machine Specification -- Kubota MX5400 4WD HST

| Specification | Detail |
| --- | --- |
| Engine | Kubota V2403 diesel -- 55.5 hp gross / 46.5 hp PTO |
| Drive | 4WD -- essential for Bicol terrain and steep hill access |
| Transmission | HST (hydrostatic) 3-range. Easier on slopes and in tight food forest conditions than gear drive |
| PTO | Live independent rear PTO -- 540 rpm |
| 3-Point Hitch | Category 1 and 2 -- accepts full range of rear attachments |
| Front Loader | LA1065 -- matched to MX series, 1,031 kg lift at pin |
| Tyres | R4 industrial -- confirm when sourcing. Hard ground, road, and wet conditions |
| Hydraulics | Open centre with third function valve -- required for 4-in-1 bucket |
| Weight | Approx. 2,100 kg without loader |

Specify HST transmission only. Gear-only is not suitable for slope work, food forest, or loader operations.

# 3. Attachment Stack

Buy in priority order. 4-in-1 front bucket and box scraper are non-negotiable from day one.

## Front -- LA1065 Loader

| Attachment | Priority | Purpose |
| --- | --- | --- |
| 4-in-1 Multi-Purpose Bucket | MUST HAVE | Scoops, dozes, clamps, spreads. Core tool for compost, earth, and materials. |
| Root Grapple 72" | HIGH | Grabs logs, brush, large debris. Food forest clearing and establishment. |
| Pallet Forks | MEDIUM | Materials handling, loading, moving feed bags and containers. |

## Rear -- 3-Point Hitch

| Attachment | Priority | Purpose |
| --- | --- | --- |
| Box Scraper / Box Blade | MUST HAVE | Road grading and maintenance after excavator cuts. Year-round road upkeep. |
| Rotary Slasher 6ft | MUST HAVE | Understorey clearing and ongoing maintenance in food forest. |
| Post Hole Digger + Auger | MUST HAVE | Fencing, tree planting, post holes. Essential daily farm tool. |
| Ripper / Subsoiler | HIGH | Breaks hard ground before road work and new planting areas. |
| BH92 Backhoe | HIGH | Rear-mount backhoe, digs 2.7m deep. Drainage, small earthworks, post holes. Quick attach/detach. Reduces excavator hire for smaller jobs. |
| Rotary Tiller | MEDIUM | Seedbed prep for regen gardening and nursery beds. |
| Compost Spreader | MEDIUM | Spreading compost across food forest and gardens. |
| Landscape Rake | LOW | Final levelling and clearing after earthworks. |

Quick hitch adapter for rear 3-point recommended -- single-person attachment changes without leaving the seat.

# 4. Sourcing Strategy -- Buy From Japan

Japan is the correct sourcing market for used Kubota machinery in the Philippines. The Philippines has an established Japan surplus import network -- the same pipeline that supplies secondhand vehicles and construction equipment. Buying Japan surplus delivers a significantly better machine at 40-60% of the cost of a new Philippine dealer unit.

## Why Japan, Not the Local Dealer

- Kubota is a Japanese brand -- deepest used stock is in the Japanese domestic market

- Japanese farm machinery is meticulously maintained and serviced -- condition is consistently better than equivalent hours elsewhere

- Established shipping pipeline Japan to Philippines via Manila and Cebu ports

- Same Kubota parts network supports Japan-sourced machines -- Naga dealer services them

- Landed Philippines price is half or less of new local dealer price for equivalent specification

## Target Specification When Sourcing

- Kubota MX5400 4WD HST -- or equivalent MX5200 / MX5100 (same engine, same attachment compatibility)

- Under 1,500 hours -- ideally under 800 hours

- LA1065 loader fitted -- or budget separately to source loader

- R4 industrial tyres -- confirm specification before purchasing

- Third function hydraulic valve fitted -- required for 4-in-1 bucket

# 5. Pricing

## Japan Auction Price -- Source Cost

| Machine | Japan Auction (JPY) | Equiv. PHP |
| --- | --- | --- |
| Kubota 50-55hp 4WD + loader, 800-1,500 hrs | Y800,000 - Y1,200,000 | PHP 320,000 - 480,000 |
| Kubota 50-55hp 4WD + loader, under 800 hrs, clean | Y1,200,000 - Y1,800,000 | PHP 480,000 - 720,000 |

## Total Landed Cost -- Naga, Camarines Sur

| Cost Component | Estimated Amount (PHP) |
| --- | --- |
| Japan auction price (machine + loader) | PHP 320,000 - 720,000 |
| Japan export agent fee | PHP 20,000 - 40,000 |
| RoRo / container shipping Japan to Manila | PHP 80,000 - 140,000 |
| Import duty + 12% VAT (agricultural machinery) | ~15% on CIF value |
| Manila port clearance and broker fees | PHP 30,000 - 50,000 |
| Freight Manila to Naga | PHP 30,000 - 60,000 |
| TOTAL LANDED BUDGET | PHP 650,000 - 1,200,000 |

## Attachments -- Sourced Locally or from Japan

| Attachment | Estimated PHP |
| --- | --- |
| 4-in-1 bucket (if not with machine) | PHP 70,000 - 130,000 |
| BH92 backhoe | PHP 380,000 - 520,000 |
| Rotary slasher 6ft | PHP 100,000 - 180,000 |
| Box scraper | PHP 45,000 - 75,000 |
| Post hole digger + auger | PHP 55,000 - 90,000 |
| Root grapple 72" | PHP 85,000 - 150,000 |
| Ripper | PHP 35,000 - 65,000 |

| Scenario | Total Budget PHP |
| --- | --- |
| Tractor + loader from Japan + priority attachments (box scraper, slasher, auger, ripper) | PHP 950,000 - 1,600,000 |
| Above + BH92 backhoe + root grapple (full kit) | PHP 1,400,000 - 2,300,000 |
| New machine from local dealer (benchmark only) | PHP 2,800,000 - 3,500,000 |

Japan sourcing saves PHP 1,500,000 - 2,000,000 compared to buying new from a Philippine dealer for equivalent specification.

# 6. Where to Source

## Japan Export Agents -- Direct

- Japan Partner: japan-partner.com -- 39 Kubota tractors listed, direct from Japanese dealers, export paperwork and pre-shipment inspection available

- Big Lemon: biglemon.kenkey.jp -- largest Japanese agricultural machinery marketplace, search Kubota 4WD loader

- TS Export: ts-export.com -- established Japan exporter since 1983, over 1,500 Kubota tractors available monthly

## Philippines Local Surplus Importers

- Kubota Tractor Japan Surplus Import Philippines -- Dasmarinas, Cavite. Facebook: @usedkubotatractor. Active importer -- call for current stock and landed prices

- Cebu and Manila surplus machinery yards -- ask the Naga Kubota dealer who they know bringing machines in locally

- OLX Philippines -- search Kubota tractor 4WD loader for private sellers and small importers

## Local Dealer (Parts and Service Only)

- Agro-Industrial Machineries Inc -- Naga City -- Kubota dealer for Camarines Sur. Use for parts, servicing, and new pricing benchmark. Confirm they service Japan-sourced machines before buying.

# 7. Inspection Checklist

Inspect before buying or pay for pre-shipment inspection through the export agent. Bring a mechanic.

## Engine

- Cold start -- check for white (coolant), black (rich/dirty), or blue (oil burning) smoke

- Check oil level and colour -- milky oil means head gasket failure

- Check coolant level and colour -- rust or low level is a warning

- Listen for knocking or rattling at idle and under load

- Check for oil leaks around head, sump, and injectors

## Hydraulics

- Raise loader to full height -- hold 2 minutes -- check for drift (cylinder seal failure)

- Test all four functions on 4-in-1 bucket -- all should move smoothly and hold position

- Inspect all hydraulic hoses for cracking, weeping, and abrasion damage

- Test 3-point hitch raise and lower under load

## Transmission and Drive

- Test full HST range forward and reverse -- smooth with no hesitation or slipping

- Engage 4WD -- drive under load, check for vibration or clunking

- Check front axle for play -- grab front wheel and rock side-to-side

## PTO

- Engage PTO at idle and full speed -- no vibration, no unusual noise

- Check PTO shaft and guards for damage -- common and expensive to fix

## Overall Condition

- Hour meter -- under 1,500 hrs good, under 800 hrs excellent

- Check frame for cracks or heavy weld repairs -- red flag

- Inspect R4 tyre condition -- replacement set is expensive

- Check ROPS (rollover protection frame) for damage or unauthorised modification

# 8. Excavator Hire Strategy

For road building, steep hill cutting, and major earthworks -- hire an excavator, do not buy. Own the machine you use every day. Hire the one you use for campaigns.

| Item | Detail |
| --- | --- |
| Machine size | 5-8 tonne with operator -- Komatsu or Hitachi preferred in Bicol |
| Hire rate (Bicol) | PHP 8,000 - 12,000 per day with operator |
| Mobilisation | Confirm transport cost from Naga or Sipocot to site before booking |
| Strategy | Plan ALL road cutting, hill benching, site prep, and drainage in one campaign. Do it once, do it properly. |
| Timing | Before September 2026 -- roads in before rainy season locks the site |

The BH92 rear backhoe on the tractor covers small daily excavation -- drainage channels, post holes, minor earthworks -- without hire costs. The hired excavator is for the big hill cuts and road campaigns only.

# 9. Summary

| Decision | Recommendation |
| --- | --- |
| Machine | Kubota MX5400 4WD HST + LA1065 loader |
| Source | Japan surplus -- not new Philippine dealer |
| Transmission | HST only -- no gear drive |
| Tyres | R4 industrial -- confirm before purchasing |
| Target hours | Under 1,500 hrs. Under 800 hrs preferred. |
| Day 1 attachments | 4-in-1 bucket, box scraper, slasher, post hole digger, ripper |
| Phase 2 attachments | BH92 backhoe, root grapple, compost spreader |
| Total budget (tractor + day 1 attachments) | PHP 950,000 - 1,600,000 |
| Excavator | Hire only. PHP 8,000-12,000/day. Campaign-based. |

Use what you have. Build what lasts. Leave something worth inheriting.